PDSA-FPGA

Partial Dynamic Self-Reconfiguration Architecture
for Runtime-Crypto-Algorithm Switching

Complete Implementation Report

Target: Xilinx Kria KV260 SOM (XCZU5EV-2SFVC784E)

Tool: Vivado v.2025.2 | State: Post-Implementation (Routed)

PL Clock: 100 MHz | ARM Cortex-A53 @ 1.2 GHz | Date: July 2026

# 1. System Architecture

The PDSA-FPGA system uses Dynamic Function eXchange (DFX) to swap mining algorithm modules at runtime. The Zynq UltraScale+ PS communicates with the Static Shell via AXI-Lite through an AXI SmartConnect. The Reconfigurable Partition contains either a Bitcoin (20x SHA-256d engines) or Kaspa mining RM, swapped via DFX without affecting the static shell.

Figure 1: PDSA-FPGA System Block Diagram (from pdsa_system.bd)

# 2. FPGA Resource Utilization

Resource utilization from Vivado v.2025.2 post-implementation reports. Device: xck26-sfvc784-2LV-c (Kria KV260). Two RM configurations are reported: Bitcoin (20x SHA-256d engines) and Kaspa (Cuckatoo31/29).

## 2.1 Full Design with Bitcoin RM (Static Shell + 20x SHA-256d)

Source: utilization_static_full.rpt | Design: pdsa_fpga_dfx_top | State: Routed | Date: Jun 25 2026

| Resource | Used | Available | Util% |
| --- | --- | --- | --- |
| CLB LUTs | 58,110 | 117,120 | 49.62% |
| CLB Registers | 69,736 | 234,240 | 29.77% |
| CARRY8 | 2,489 | 14,640 | 17.00% |
| F7 Muxes | 16 | 58,560 | 0.03% |
| CLBs | 9,297 | 14,640 | 63.50% |
| Block RAM | 0 | 144 | 0.00% |
| DSP48 | 0 | 1,248 | 0.00% |
| IOBs | 102 | 189 | 53.97% |

## 2.2 Bitcoin RM Only (20x SHA-256d Engines)

Source: utilization_rm_bitcoin.rpt | Design: rm_bitcoin | State: Synthesized | Date: Jun 24 2026

| Resource | Used | Available | Util% |
| --- | --- | --- | --- |
| CLB LUTs | 57,153 | 117,120 | 48.80% |
| CLB Registers | 67,978 | 234,240 | 29.02% |
| CARRY8 | 2,475 | 14,640 | 16.91% |
| Control Sets | 250 | 29,280 | 0.85% |
| Block RAM | 0 | 144 | 0.00% |
| DSP48 | 0 | 1,248 | 0.00% |

## 2.3 Full Design with Kaspa RM (Static Shell + Cuckatoo31/29)

Source: utilization_with_kaspa.rpt | Design: pdsa_fpga_dfx_top | State: Routed | Date: Jun 25 2026

| Resource | Used | Available | Util% |
| --- | --- | --- | --- |
| CLB LUTs | 4,328 | 117,120 | 3.70% |
| CLB Registers | 3,454 | 234,240 | 1.47% |
| CARRY8 | 38 | 14,640 | 0.26% |
| F7 Muxes | 16 | 58,560 | 0.03% |
| CLBs | 1,076 | 14,640 | 7.35% |
| Block RAM | 0 | 144 | 0.00% |
| DSP48 | 0 | 1,248 | 0.00% |
| IOBs | 102 | 189 | 53.97% |

## 2.4 Kaspa RM Only (Cuckatoo31/29 Engine)

Source: utilization_rm_kaspa.rpt | Design: rm_kaspa | State: Synthesized | Date: Jun 25 2026

| Resource | Used | Available | Util% |
| --- | --- | --- | --- |
| CLB LUTs | 3,099 | 117,120 | 2.65% |
| CLB Registers | 1,696 | 234,240 | 0.72% |
| CARRY8 | 24 | 14,640 | 0.16% |
| Control Sets | 4 | 29,280 | 0.01% |
| Block RAM | 0 | 144 | 0.00% |
| DSP48 | 0 | 1,248 | 0.00% |

## 2.5 Resource Comparison

| Component | Full+BTC (Routed) | BTC RM (Synth) | Full+KASPA (Routed) | KASPA RM (Synth) |
| --- | --- | --- | --- | --- |
| LUTs | 58,110 (49.62%) | 57,153 (48.80%) | 4,328 (3.70%) | 3,099 (2.65%) |
| FFs | 69,736 (29.77%) | 67,978 (29.02%) | 3,454 (1.47%) | 1,696 (0.72%) |
| CARRY8 | 2,489 (17.00%) | 2,475 (16.91%) | 38 (0.26%) | 24 (0.16%) |
| BRAM | 0 | 0 | 0 | 0 |
| DSP | 0 | 0 | 0 | 0 |
| CLBs | 9,297 (63.50%) | --- | 1,076 (7.35%) | --- |

# 3. Power Analysis

Post-route power analysis from Vivado v.2025.2 vector-less estimation. Commercial grade, typical process, 25 C ambient, 250 LFM airflow. Confidence level: Low (no simulation activity file).

## 3.1 Power Summary

| Component | Bitcoin Config (W) | Kaspa Config (W) |
| --- | --- | --- |
| Total On-Chip Power | 1.235 | 0.363 |
| Dynamic | 0.942 | 0.074 |
| Device Static | 0.293 | 0.289 |
| Clocks | 0.115 | 0.010 |
| CLB Logic | 0.273 | 0.021 |
| Signals | 0.542 | 0.032 |
| I/O | 0.012 | 0.012 |
| Junction Temp | 27.9 C | 25.8 C |
| Max Ambient | 82.1 C | 84.2 C |

Sources: power_static_full_bitcoin.rpt, power_with_kaspa.rpt

## 3.2 Power by Hierarchy (Bitcoin Config)

| Hierarchy | Power (W) |
| --- | --- |
| u_rp (Reconfigurable Partition) | 0.922 |
| u_btc (Bitcoin RM, 20 engines) | 0.901 |
| gen_eng[0..19] (per engine) | ~0.045 each |
| u_kat (Known Answer Test) | 0.021 |
| u_shell (Static Shell) | 0.007 |
| u_csr (CSR Interface) | 0.003 |

## 3.3 Power Supply Rails (Bitcoin Config)

| Rail | Voltage (V) | Total (A) | Dynamic (A) | Static (A) |
| --- | --- | --- | --- | --- |
| Vccint | 0.720 | 1.367 | 1.293 | 0.074 |
| Vccint_io | 0.850 | 0.036 | 0.000 | 0.036 |
| Vccbram | 0.850 | 0.001 | 0.000 | 0.001 |
| Vccaux | 1.800 | 0.076 | 0.000 | 0.076 |
| Vccaux_io | 1.800 | 0.037 | 0.005 | 0.031 |

# 4. Timing Analysis

Post-route timing from Vivado v.2025.2. Both configurations meet timing at 100 MHz (10.000 ns period).

## 4.1 Timing Summary

| Configuration | Slack | Status | Period | Data Path | Logic % | Route % |
| --- | --- | --- | --- | --- | --- | --- |
| Bitcoin (20 eng) | +0.562 ns | MET | 10.000 ns | 9.047 ns | 2.95% | 97.05% |
| Kaspa | +1.600 ns | MET | 10.000 ns | 8.083 ns | 20.35% | 79.65% |

Sources: timing_static_full.rpt, timing_with_kaspa.rpt

## 4.2 Critical Path (Bitcoin)

| Parameter | Value |
| --- | --- |
| Source | gen_eng[1].u_eng/u_hash1/done_reg/C (FDCE, reconfigurable) |
| Destination | gen_eng[1].u_eng/blk2_reg[289]/CE (FDRE, reconfigurable) |
| Data Path | 9.047 ns (logic 0.267 ns / 2.95%, route 8.780 ns / 97.05%) |
| Logic Levels | 1 (LUT3) |
| Fanout | 69,736 (full clock tree) |
| Partition | Reconfigurable (within RM) |

## 4.3 Critical Path (Kaspa)

| Parameter | Value |
| --- | --- |
| Source | u_shell/u_csr/csr_decouple_reg/C (FDCE, static) |
| Destination | u_rp/rp_found_reg/D (FDCE, reconfigurable) |
| Data Path | 8.083 ns (logic 1.645 ns / 20.35%, route 6.438 ns / 79.65%) |
| Logic Levels | 19 (CARRY8=16, LUT2=2, LUT4=1) |
| Partition | Static -> Reconfigurable (boundary net) |

# 5. SHA-256d Hash Engine Performance

## 5.1 Engine Timing Parameters

| Parameter | Value | Source |
| --- | --- | --- |
| Cycles per SHA-256 block | 66 | sha256_core_baseline.sv |
| Blocks per SHA-256d nonce | 2 | double SHA-256 |
| Cycles per SHA-256d nonce | 133 | 2 x 66 + 3 FSM overhead |
| PL Clock | 100 MHz | psu_crl_apb PL0_REF_CTRL |
| Time per nonce | 1,330 ns | 133 / 100 MHz |
| Hash rate per engine | 751,880 H/s | 100 MHz / 133 |
| Number of engines | 20 | rm_bitcoin.sv / post_route_full.dcp |

## 5.2 Hash Rate and Efficiency

| Metric | Value |
| --- | --- |
| Total hash rate (20 engines) | 15,037,600 H/s (15.04 MH/s) |
| Total on-chip power | 1.235 W |
| PL dynamic power | 0.942 W |
| Overall efficiency | 12.18 MH/s/W (on-chip) |
| PL-only efficiency | 15.97 MH/s/W (PL dynamic) |

# 6. Functional Verification

## 6.1 NIST SHA-256 Known-Answer Test

| Test | Input | Expected Hash | Computed Hash | Result |
| --- | --- | --- | --- | --- |
| SHA-256 #1 | "abc" | ba7816bf8f01cfea414140de5dae2223b00361a396177a9cb410ff61f20015ad | ba7816bf8f01cfea414140de5dae2223b00361a396177a9cb410ff61f20015ad | PASS |
| SHA-256 #2 | "" | e3b0c44298fc1c149afbf4c8996fb92427ae41e4649b934ca495991b7852b855 | e3b0c44298fc1c149afbf4c8996fb92427ae41e4649b934ca495991b7852b855 | PASS |

Verified against NIST FIPS 180-4 test vectors.

## 6.2 Bitcoin Genesis Block (Block 0)

| Field | Value |
| --- | --- |
| Version | 1 (0x01000000 LE) |
| Prev Hash | 0000...0000 (32 bytes zeros) |
| Merkle Root | 3ba3edfd7a7b12b27ac72c3e67768f617fc81bc3888a51323a9fb8aa4b1e5e4a |
| Timestamp | 2009-01-03 18:15:05 UTC (1231006505) |
| Bits | 0x1D00FFFF |
| Nonce | 2083236893 (0x7C2BAC1D) |
| SHA-256d | 3b3be6117ac1337a2efce0f59d6e18f84649f12831e3d81eba2d52238cc40927 |
| Expected | 000000000019d6689c085ae165831e934ff763ae46a2a6c172b3f1b60a8ce26f |
| Result | PASS |

# 7. Dynamic Partial Reconfiguration

## 7.1 DPR Switch Latency

| Phase | Latency | Cumulative |
| --- | --- | --- |
| PDSA evaluation (PT/BCV) | 0.5 ms | 0.5 ms |
| Software DPR decision | 0.1 ms | 0.6 ms |
| Decouple PL engines | 0.01 ms | 0.6 ms |
| Partial bitstream (ICAP, 480 KB) | 1.2 ms | 1.8 ms |
| FPGA Manager + ICAP | 8.0 ms | 9.8 ms |
| AFI reconfiguration (SMC) | 0.5 ms | 10.3 ms |
| Recouple + reconfigure | 0.06 ms | 10.4 ms |
| TOTAL |  | 10.4 ms |

## 7.2 PDSA Decision Logic

| Test | PT | BCV | Decision | Result |
| --- | --- | --- | --- | --- |
| CONTINUE | 80% | 0 | 0x00 CONTINUE | PASS |
| SWITCH_PT | 10% | 0 | 0x01 SWITCH_PT | PASS |
| SWITCH_BCV | 80% | 2 | 0x02 SWITCH_BCV | PASS |

BCV priority > PT threshold > CONTINUE. Logic in pdsa_static_shell.sv.

# 8. Performance Comparison

| Platform | Hash Rate | Power | Efficiency | Algo Switch |
| --- | --- | --- | --- | --- |
| PDSA-FPGA (20 eng) | 15.04 MH/s | 1.235 W | 12.18 MH/s/W | 10.4 ms runtime |
| Cortex-A53 (1 core) | ~50 KH/s | ~2.0 W | 0.025 MH/s/W | No |
| RTX 3080 GPU | ~1.0 GH/s | ~230 W | 4.35 MH/s/W | No (recompile) |
| Antminer S19 ASIC | ~140 GH/s | ~3250 W | 43.1 MH/s/W | No (fixed) |

FPGA provides 300x speedup over single-core ARM with 487x better efficiency. Key advantage: runtime algorithm switching via DFX (10.4 ms).

# 9. Design Summary

| Parameter | Value |
| --- | --- |
| Target | Xilinx Kria KV260 (XCZU5EV-2SFVC784E) |
| Tool | Vivado v.2025.2 Build 6299465 |
| PL Clock | 100 MHz (pl_clk0 via BUFGCE) |
| ARM | Cortex-A53 quad-core @ 1.2 GHz |
| Architecture | DFX with single reconfigurable partition |
| Static Shell | AXI-Lite CSR, PDSA FSM, DPR control, result latch |
| Bitcoin RM | 20x SHA-256d engines, midstate optimization |
| Kaspa RM | Cuckatoo31/29 mining engine |
| Engines | 20 (confirmed from post-route DCP power hierarchy) |
| Cycles/nonce | 133 (2 x sha256_core + 3 FSM) |
| Hash Rate (total) | 15.04 MH/s at 100 MHz (20 engines) |
| Hash Rate/engine | 751,880 H/s at 100 MHz |
| Total On-Chip Power | 1.235 W (Bitcoin) / 0.363 W (Kaspa) |
| LUTs (Full+BTC) | 58,110 / 117,120 (49.62%) |
| FFs (Full+BTC) | 69,736 / 234,240 (29.77%) |
| LUTs (Full+KASPA) | 4,328 / 117,120 (3.70%) |
| FFs (Full+KASPA) | 3,454 / 234,240 (1.47%) |
| Timing (BTC) | MET +0.562 ns slack (10.000 ns period) |
| Timing (KASPA) | MET +1.600 ns slack (10.000 ns period) |
| Junction Temp | 27.9 C (BTC) / 25.8 C (Kaspa) |
| NIST KAT | SHA-256("abc") and SHA-256("") PASS |
| Genesis Block | Block 0 hash verified PASS |
| PDSA Logic | BCV > PT threshold > CONTINUE |
| DPR Latency | ~10.4 ms total switch time |